Architecture Design Document

POC v2 — Gemini-Brain Robotic Inspection in MuJoCo / robosuite

Project path: C:\huminoid     |     POC v2     |     June 2026

# 1. Executive Summary

POC v2 upgrades the humanoid inspection demo from a single-purpose YOLOv8 detector into a modern two-layer robot architecture that mirrors how leading humanoid companies build their systems: a hosted Vision-Language-Model (VLM) 'brain' for perception and decision-making, paired with a separate action layer for motor control. The operator types a natural-language instruction such as "throw all the defective parts into the reject bin"; a hosted Gemini 2.0 Flash brain looks at the simulated work cell, decides which parts are defective and what to do, and a reliable robosuite controller executes the pick-and-place inside MuJoCo.

The goal of this document is to define an architecture that produces a working, repeatable demo in MuJoCo on commodity hardware (AMD RX 580, no CUDA), while remaining honest about which components are reliable today versus which are best-effort showcases.

# 2. Why v2 — Limits of v1

| v1 Limitation | Impact | v2 Resolution |
| --- | --- | --- |
| YOLOv8 is the only 'intelligence' | Every decision is hardcoded in an FSM; no reasoning | Gemini VLM brain reasons over the scene and the instruction |
| Required ~2h CPU training on synthetic data | Slow iteration; brittle to scene changes | Zero-shot hosted brain — no training to change behaviour |
| Hardcoded keyframe motor layer | Not a real policy; no generalization | Reliable OSC controller + optional real VLA (Octo / GR00T) |
| Placeholder capsule scene | Looks like a prototype | robosuite PickPlace BinsArena — professional, ready-made |
| Behaviour fixed at code level | New rule = code edit + retrain | Behaviour changes by editing one English sentence |

# 3. Industry Landscape — The VLM-Brain + Action-Model Pattern

Research into the most successful humanoid programs of 2025-2026 shows a single converging architecture: a VLM backbone for high-level reasoning, paired with a dedicated action model for low-level motor control. POC v2 deliberately adopts this same split. Notably, Gemini — our chosen brain — is the very model Google ships in Gemini Robotics.

| Organization | Brain (VLM) | Action layer | Relevance to POC v2 |
| --- | --- | --- | --- |
| Google DeepMind | Gemini Robotics (Gemini VLM) | Diffusion action decoder | Gemini is the same family we use as the brain |
| Figure AI | Helix (in-house VLM) | Onboard motor policy | Full-stack; drives Figure 02/03 hands+arms |
| NVIDIA | GR00T N1.5 VLM backbone | Diffusion Transformer | Open, customizable humanoid foundation model |
| Physical Intelligence | pi-0 VLM | Flow-matching action expert | General-purpose manipulation VLA |
| Tesla | In-house vision stack | In-house motor control | Optimus; 28-DOF full-body control |

Sources: NVIDIA GR00T N1 / N1.5 announcements (Mar / May 2025); Google DeepMind Gemini Robotics; Figure AI Helix; Physical Intelligence pi-0. See evsint.com 'Top Robotics Foundation Model & Embodied AI Companies 2026' and the awesome-physical-ai survey.

# 4. System Architecture

The system is organized into four layers. Data flows down (camera -> brain -> decision -> action -> robot); the only operator input is a natural-language task instruction.

Figure 1 — POC v2 layered architecture

## 4.1 Layer responsibilities

| Layer | Component | Responsibility | Hosting |
| --- | --- | --- | --- |
| Simulation | MuJoCo + robosuite PickPlace | Physics, rendering, ground-truth object poses | Local |
| Brain | Gemini 2.0 Flash | Perceive scene + decide (pixels -> JSON Decision) | Hosted (cloud) |
| Decision | Decision dataclass | Typed contract between brain and action | In-process |
| Action | Scripted OSC (default) | Reliable reach / grasp / place | Local |
| Action (showcase) | Octo / GR00T | Real VLA motor control (best-effort) | Local / Hosted |
| Robot | Panda arm | Executes the sort into the reject bin | Local (sim) |

# 5. Component Specifications

## 5.1 Brain abstraction

All brains implement a common interface and return a typed Decision, so modes are fully interchangeable from the demo loop:

@dataclass

class Decision:

target_object:  str     # robosuite object name, e.g. 'Cereal'

classification: str     # 'good' | 'defect'

action:         str     # 'pick_reject' | 'pass' | 'watch'

confidence:     float

reasoning:      str     # plain-English 'why' (shown on the HUD)

class Brain(ABC):

def decide(self, frame_bgr, instruction, scene_context) -> list[Decision]: ...

| Brain mode | Backing model | Hosting | Notes |
| --- | --- | --- | --- |
| gemini (default) | Gemini 2.0 Flash | Hosted | Free tier; video-capable; primary path |
| groot | NVIDIA GR00T N1.5 | Hosted* | VLA; endpoint must be verified (see 7) |
| yolo | v1 YOLOv8n detector | Local | Reuses vision/detector.py for comparison |

## 5.2 Action abstraction

class ActionLayer(ABC):

def execute(self, env, decision: Decision) -> bool: ...   # success?

def reset(self): ...

| Action mode | Mechanism | Reliability | Role |
| --- | --- | --- | --- |
| scripted (default) | robosuite OSC_POSE waypoints | High | Backbone — demo always works |
| octo | Octo VLA (JAX, CPU) | Low (jittery) | Showcase: real generalist policy |
| groot | GR00T N1.5 action head | Unverified | Showcase: humanoid foundation model |

The scripted OSC layer uses ground-truth object poses from the simulator for MOTION only; the brain still performs all PERCEPTION and DECISION-making from pixels. This cleanly separates 'where things are' (trivial in sim) from 'what to do' (the interesting part).

## 5.3 Scene mapping — PickPlace as inspection

| robosuite element | Inspection meaning |
| --- | --- |
| Center bin with 4 objects (Can, Cereal, Milk, Bread) | Parts arriving for inspection |
| Gemini classification per object | good vs defect call |
| One target container | Reject bin for defective parts |
| Panda arm + gripper | The inspecting robot |

# 6. Control Loop & Data Flow

env = suite.make('PickPlace', robots='Panda',

use_camera_obs=True, camera_names=['frontview'],

has_offscreen_renderer=True, control_freq=20)

obs = env.reset()

while not done:

frame = obs['frontview_image']           # RGB from MuJoCo

if time_to_replan():                      # THROTTLED brain call

decisions = brain.decide(frame, instruction, scene_context)

for d in decisions if d.action == 'pick_reject':

action_layer.execute(env, d)          # OSC reach->grasp->place

obs, _, done, _ = env.step(noop_or_action)

hud = draw_overlay(frame, status, reasoning=d.reasoning)

Brain is called only at decision points (per re-plan), never per frame — respects Gemini free-tier rate limits.

The Decision.reasoning string is rendered on the HUD so viewers see WHY the robot acted.

Scripted OSC executes a fixed waypoint sequence: approach -> descend -> grasp -> lift -> move-to-reject -> release.

# 7. Hardware Limits & Hosting Strategy

| Constraint | Consequence | Strategy |
| --- | --- | --- |
| AMD RX 580, no CUDA | Cannot run 7B VLAs locally on GPU | Brain is hosted (Gemini / GR00T cloud) |
| 16 GB RAM, CPU only | Octo runs but slowly (~seconds/step) | Octo is a showcase mode, throttled |
| Gemini free-tier limits | Rate caps on requests/min | Call brain per-decision, cache last plan |
| Windows + JAX | Octo/JAX install can be fragile | Octo isolated in optional requirements-octo.txt |

# 8. Honest Reality Checks

Octo zero-shot in robosuite grasps unreliably without fine-tuning (needs demo trajectories + GPU). It is a SHOWCASE only; the scripted OSC layer is what makes the demo reliable.

GR00T is primarily open weights (nvidia/GR00T-N1.5-3B) on HuggingFace, not guaranteed to be a hosted NIM API. The build pass must VERIFY a hosted endpoint using NVIDIA_API_KEY; if absent, GR00T mode degrades gracefully and Gemini remains the working path.

GR00T is technically a VLA action model, so it is wired alongside Octo as a foundation-model policy mode — it is NOT a drop-in replacement for the Gemini decision brain.

The reliable, always-demoable path is: --brain gemini --action scripted.

# 9. Path to a Working MuJoCo Demo

Planned module layout for the build pass (kept parallel to v1; v1 stays intact):

C:\huminoid\

robosuite_demo.py          # v2 main entry (env loop + HUD + CLI)

config.py                  # instruction, defect mapping, throttle, model IDs

brains\  base.py  gemini_brain.py  groot_brain.py  yolo_brain.py

action\  base.py  scripted_osc.py  octo_policy.py  groot_policy.py

## 9.1 Run modes

# Reliable path (recommended for any live demo)

python robosuite_demo.py --brain gemini --action scripted \

--instruction "throw all the defective parts into the reject bin"

# Headless (save annotated frames to output\)

python robosuite_demo.py --brain gemini --action scripted --no-viewer

# Showcase: real generalist VLA driving the arm

python robosuite_demo.py --brain gemini --action octo

# Comparison: original YOLO brain

python robosuite_demo.py --brain yolo --action scripted

## 9.2 Verification checklist

| Check | Pass condition |
| --- | --- |
| Env smoke test | PickPlace creates, resets, renders one frame to output\env_smoke.jpg |
| Brain test | Gemini returns valid Decision list with classifications + reasoning |
| Action test | Scripted OSC places one known object inside the reject container |
| Full reliable demo | Runs to completion headless; prints inspected/defects/sorted metrics |
| Live viewer | robosuite window opens and the arm visibly sorts defects |
| Showcase modes | octo loads & steps without crashing; groot degrades cleanly if no endpoint |

# 10. Environment Setup

cd C:\huminoid

.venv\Scripts\activate

pip install robosuite google-generativeai

# Gemini key (free at aistudio.google.com)

$env:GEMINI_API_KEY = "AIza..."

# NVIDIA key (already configured) — only for GR00T modes

$env:NVIDIA_API_KEY = "nvapi-..."

# Optional showcase only:

pip install -r requirements-octo.txt   # octo, jax[cpu], flax